Flash Flood Vulnerability and Forecasting System  Hudson County

User Manual

Table of Contents

1. Introduction

2. Objectives and Goals

3. System Overview

4. Static Flood Vulnerability Mapping

5. Dynamic Flood Risk Modeling

6. Temporal Analysis (ETA and Maximum Risk)

7. StreetLevel Risk Aggregation

8. System Workflow

9. Website / Application Interface

10. Data Inputs and Outputs

11. Installation and Setup

12. Usage Instructions

13. Visualization and Interpretation

14. Limitations

15. Future Work

16. Batch Processing

17. External API

18. Alert Notification

1. Introduction

Flash floods are rapid onset hydrological events that develop within minutes to hours, particularly in urbanized watersheds and small drainage basins. Unlike riverine flooding, these events are highly localized and difficult to predict using traditional basinscale warning systems.

Hudson County, New Jersey, presents a complex flood environment due to dense urban development, impervious surfaces, and constrained drainage networks. Existing flood mapping approaches such as FEMA flood zones provide static, long-term risk but do not indicate when or where flooding will occur during a specific storm.

This system addresses that limitation by combining terrain based vulnerability analysis with real-time environmental inputs to generate streetlevel flood risk forecasts. The system produces spatially detailed outputs that support operational decisionmaking, including road closures and emergency response planning.

Current warning systems focus mostly on large rivers and often miss localized events. Factors like drainage capacity, soil saturation, and impervious surfaces interact in complex ways that simple thresholds cannot capture. As a result, warnings often come too late or not at all. This system directly addresses these gaps by providing streetlevel predictions that enable real-time decision making.

2. Objectives and Goals

The system is designed to achieve the following:

- Analyze terrain, land cover, and drainage features to understand baseline flood susceptibility

- Integrate rainfall and soil moisture data to capture dynamic environmental conditions

- Provide streetlevel flood predictions that support operational response

- Enable real-time decision making for emergency managers and infrastructure operators

The primary goal is to generate forecasts showing:

- Where flooding occurs (spatial extent and location)

- How deep water gets (risk intensity and severity)

- When flooding starts (estimated time of arrival)

3. System Overview

The system consists of three integrated components:

1. Static Vulnerability Model

- A terrain based raster that represents inherent flood susceptibility

- Uses elevation, slope, and flow accumulation derived from DEM

- Timeinvariant layer that serves as the foundation for all dynamic calculations

2. Dynamic Risk Model

- A time-dependent model that incorporates rainfall and soil moisture

- Estimates flood risk on an hourly basis for 48hour forecast horizons

- Generates risk = static vulnerability × (rain / MAX_RAIN) × soil

3. Web Application

- An interactive platform built on Leaflet mapping library

- Visualizes static and dynamic outputs with toggle controls

- Supports user queries, watchlists, address search, and realtime risk information

The system operates as a pipeline where static data forms the baseline and dynamic inputs drive temporal variation. The entire pipeline from rainfall fetch to final GeoJSON output typically completes within 10 to 15 minutes.

4. Static Flood Vulnerability Mapping

The static flood vulnerability map represents areas naturally prone to flooding based on terrain characteristics. This layer is time invariant and serves as the foundation for all dynamic calculations.

4.1 Input Data

| Input | Description | Source |
| --- | --- | --- |
| DEM | Digital Elevation Model (terrain elevation) | Geospatial survey data |
| Slope | Slope raster expressing the rate of elevation change (in degrees) for each pixel, derived from the DEM using GRASS r.watershed. | Generated from hudson_dem_clip during GRASS r.watershed terrain processing in QGIS. |
| Flow accumulation | Flow accumulation raster representing the number of upstream cells draining into each pixel, derived from the DEM using GRASS r.watershed. | Generated from hudson_dem_clip DEM using GRASS GIS hydrology tools in QGIS. |

4.2 Methodology

Step 1: LowLying Area Identification

- Formula: Lowland_risk = (elevation < threshold) × 1

- Purpose: Identifies terrain depressions where water naturally accumulates

- Rationale: Based on gravitational pooling principles

Step 2: Slope Normalization

- Formula: Slope_norm = (slope  min) / (max  min)

- Purpose: Converts slope into a normalized risk scale (0–1)

- Rationale: Flat surfaces promote water stagnation; steep slopes facilitate runoff

Step 3: Flow Accumulation Normalization

- Formula: FlowAcc_norm = ln(flow_acc + 1) / ln(max_value + 1)

- Purpose: Reduces skewness in flow accumulation distribution

- Rationale: Highlights drainage pathways and convergence zones

Step 4: Weighted Integration

- Formula: Static_FV = 0.4 × FlowAcc_norm + 0.3 × Slope_norm + 0.3 × Lowland_risk

Weight Justification:

- Flow Accumulation (40%) : Strongest indicator of flood risk; measures upstream drainage area

- Slope (30%) : Controls runoff velocity and ponding potential

- Lowland Risk (30%) : Represents natural pooling zones where water collects

4.3 Output

| Property | Value |
| --- | --- |
| File name | static_fv_10m.tif |
| Resolution | 10 meters |
| Value range | 0 (low risk) to 1 (high risk) |
| Format | GeoTIFF |

4.4 Interpretation

| Value Range | Interpretation |
| --- | --- |
| 0.0 – 0.25 | low flood susceptibility |
| 0.25 – 0.55 | Moderate susceptibility |
| 0.4 – 0.8 | High susceptibility |

Static Flood Vulnerability Map

5. Dynamic Flood Risk Modeling

Dynamic risk represents flood likelihood under current or forecast environmental conditions. The model produces 48 hourly risk rasters representing forecast evolution over time.

5.1 Risk Formula

risk = static × (rain / MAX_RAIN) × soil

Where:

- static = Static vulnerability value (0–1 scale)

- rain = Rainfall intensity in millimeters per hour

- MAX_RAIN = 75 mm (reference maximum rainfall threshold)

- soil = Volumetric water content (soil moisture)

5.2 Model Behavior

- Risk increases proportionally with rainfall intensity

- Risk increases with higher soil saturation levels

- Static vulnerability amplifies spatial differences between locations

- Multiplicative logic requires all three factors to align for significant flooding

5.3 Rainfall Normalization

- Formula: rain / MAX_RAIN where MAX_RAIN = 75 mm

- Values below 75 mm produce proportional risk reduction

- Values at or above 75 mm drive risk toward static vulnerability ceiling

5.4 Soil Moisture Impact

- Soil values represent volumetric water content from NASA POWER

- Range: 0 (completely dry) to near 1 (fully saturated)

- Saturated soils dramatically increase flood risk even with moderate rainfall

5.3 Outputs

| Output | Format | Description |
| --- | --- | --- |
| risk_YYYYMMDD_HH.tif | GeoTIFF | 48 hourly risk rasters |
| File naming convention | Timestamp-based | Year, month, day, hour of forecast validity |

6. Temporal Analysis (ETA and Maximum Risk)

Temporal analysis transforms the 48 hourly risk rasters into two highly actionable products for operational decision making.

6.1 Estimated Time of Arrival (ETA)

ETA identifies the first hour when flooding becomes significant at each location.

| Parameter | Value |
| --- | --- |
| Threshold | risk ≥ 0.05 |
| Method | Scan 48 hours chronologically, record first exceedance |
| Output file | eta.tif |
| Value type | Hour index (1–48) |
| Null values | Pixels never exceeding threshold |

Operational significance: Enables prioritization of evacuation routes and resource deployment based on flooding sequence.

6.2 Maximum Risk

Maximum risk captures the peak flood severity expected at each location over the full forecast horizon.

| Parameter | Value |
| --- | --- |
| Method | Pixelwise maximum across all 48 hourly rasters |
| Output file | max_risk.tif |
| Value range | 0–1 (same scale as static vulnerability) |

Operational significance: Distinguishes between brief, shallow flooding and prolonged, deep inundation.

Street Level risk ETA based on Hurricane Ida

Maximum Risk Map Image

7. StreetLevel Risk Aggregation

Raster outputs are converted into streetlevel data for operational use by emergency responders and transportation agencies.

7.1 Inputs

| Input | Format | Source |
| --- | --- | --- |
| eta.tif | GeoTIFF | Temporal analysis output |
| max_risk.tif | GeoTIFF | Temporal analysis output |
| Road network | GeoJSON | Hudson County transportation data |

7.2 Processing Steps

1. Load road network GeoJSON containing all county streets

2. For each road segment, extract all intersecting raster pixels

3. Calculate maximum risk value across intersecting pixels

4. Record ETA hour corresponding to maximum risk pixel

5. Write values as new properties to each road segment

7.3 Output

| Property | Value |
| --- | --- |
| File name | street.geojson |
| Size | Typically under 50 MB |
| Geometry | Linestring (road centerlines) |
| Attributes | max_risk, eta_hour (plus original road attributes) |

Operational use: Enables direct mapping of flood risk onto transportation infrastructure for route planning and road closure decisions.

8. System Workflow

The system follows a sequential pipeline with eight distinct stages.

8.1 Pipeline Stages

| Stage | Operation | Output |
| --- | --- | --- |
| 1 | Generate static vulnerability raster | static_fv_10m.tif |
| 2 | Fetch rainfall data (OpenMeteo API) | Rainfall time series (48 hours) |
| 3 | Retrieve soil moisture data (NASA POWER) | Soil moisture time series (48 hours) |
| 4 | Compute 48 hourly dynamic risk rasters | risk_YYYYMMDD_HH.tif (48 files) |
| 5 | Calculate ETA raster | eta_YYYYMMDD_HH.tif |
| 6 | Calculate maximum risk raster | max_risk.tif |
| 7 | Aggregate results to streetlevel GeoJSON | street.geojson |
| 8 | Serve products through web application | Interactive map interface |

8.2 Directory Structure

FlashFlood/

├── scripts/                 		 Processing scripts

├── data_dynamic_raw/         	Raw API data (rainfall, soil)

├── data_dynamic_processed/   	Generated rasters and GeoJSON

├── website/                  		Web application files

└── config.yaml               		Configuration parameters

8.3 Execution Time

- Typical pipeline completion: 10–15 minutes

- Dependent on network latency and raster processing complexity

- Progress tracked through status API with 2second polling

___________________________________________________________________

9. Website Interface

The system includes an interactive web interface built on Leaflet for visualization and analysis.

9.1 Simple View Features

| Feature | Description |
| --- | --- |
| Mapbased interaction | Click anywhere to query risk values |
| Risk popup | Displays static vulnerability, dynamic risk, ETA, max risk |
| Address search | Nominatim geocoding for addresses and landmarks |
| Watchlist | Save locations for monitoring |
| Mode selector | Switch between Static, Dynamic, Forecast, and Historical views |

9.2 Advanced View Features

| Feature | Description |
| --- | --- |
| Weather panel | Displays current rainfall and soil moisture |
| Risk distribution visualization | Histogram distribution |
| Watchlist management | Add, remove, and organize saved locations |
| Pipeline execution controls | Run forecasts manually |
| Time slider | Scroll through 48hour forecast timeline |
| Data download links | Export rasters and GeoJSON |

9.3 Dynamic Risk Engine Modes

| Mode | Description |
| --- | --- |
| Dynamic Mode | Uses real-time rainfall and soil moisture from Open-Meteo and NASA POWER |
| Forecast Mode | Uses 48-hour pre-generated rasters; run from the Forecast Engine panel |

9.4 Risk Color Scale

| Level | Score | Color | What to expect |
| --- | --- | --- | --- |
| None | 0–20% | 22C55E | No flooding expected |
| Low | 20–40% | 84CC16 | Minor puddles possible |
| Moderate | 40–60% | EAB308 | Shallow street water (5–15 cm) |
| High | 60–80% | F97316 | Roads may be impassable (15–30 cm) |
| Very High | 80–100% | EF4444 | Severe flooding risk (30–60+ cm) |

10. Data Inputs and Outputs

10.1 Input Data

| Input | Source | Format | Resolution |
| --- | --- | --- | --- |
| DEM | Geospatial survey | GeoTIFF | 10 meters |
| Slope | Derived from DEM | GeoTIFF | 10 meters |
| Flow accumulation | Derived from DEM | GeoTIFF | 10 meters |
| Rainfall | OpenMeteo API | JSON/Time series | Hourly |
| Soil moisture | NASA POWER API | JSON/Time series | Hourly |
| Road network | County GIS | GeoTIFF | Vector |

10.2 Output Data

| Output | Format | File pattern | Size estimate |
| --- | --- | --- | --- |
| Static vulnerability | GeoTIFF | static_fv_10m.tif | ~500 MB |
| Hourly risk rasters (48) | GeoTIFF | risk_YYYYMMDD_HH.tif | ~57 GB total |
| ETA raster | GeoTIFF | eta.tif | ~500 MB |
| Maximum risk raster | GeoTIFF | max_risk.tif | ~500 MB |
| Street forecast | GeoTIFF | street.geojson | ~50 MB |

10.3 External API Endpoints

| API | Purpose | Endpoint |
| --- | --- | --- |
| Open-Meteo | Rainfall forecasts | https://api.openmeteo.com/v1/forecast |
| NASA POWER | Soil moisture | https://power.larc.nasa.gov/api |
| Nominatim | Openstreetmap + address search | https://nominatim.openstreetmap.org/search |

11. Installation and Setup

11.1 System Requirements

| Component | Requirement |
| --- | --- |
| Operating system | Windows 10+, macOS 10.15+, or Linux (Ubuntu 20.04+) |
| RAM | 8 GB minimum (16 GB recommended when running the forecast pipeline) |
| Storage | 2 GB available |
| CPU | Any modern processor |
| Python | 3.9 or newer |
| Node.js | 18 or newer (npm included) |
| Browser | Chrome, Firefox, or Edge (current version) |

11.2 Backend Installation

Bash

cd server

pip install pandas rasterio flask flask-cors numpy requests pyproj geopandas

python server.py

11.3 Frontend Installation

Bash

npm install

npm run dev

11.4 Configuration File (config.yaml)

| Parameter | Value | Description |
| --- | --- | --- |
| MAX_RAIN | 75 | Maximum rainfall threshold (mm/hr) |
| ETA_threshold | 0.05 | Risk threshold for flooding onset |
| forecast_hours | 48 | Number of hourly forecasts |
| dem_path | data/dem/hc_dem_10m.tif | DEM file location |
| roads_path | data/roads/hc_roads.geojson | Road network location |

11.5 Access

- Web application: http://localhost:5173

- API server: http://localhost:5000

12. Usage Instructions

12.1 Basic Workflow

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | Open browser to http://localhost:5173 | Map displays static vulnerability view |
| 2 | Navigate to Forecast view | Additional controls become visible |
| 3 | Click Run Forecast Pipeline (Heartbeat Moni) | System fetches weather data and generates 48-hour forecast |
| 4 | Monitor progress (polls every 2 seconds) | Status updates show current stage |
| 5 | Click any location on map | Popup displays risk score and ETA |
| 6 | Use search bar for address | Map pans and zooms to location |
| 7 | Save locations to watchlist | Locations persist across sessions |
| 8 | Monitor alerts during active storms | Banner notifications for High/Very High risk |

12.2 Watchlist Management

| Action | Method |
| --- | --- |
| Add location | Click star icon in risk popup |
| Remove location | Click star again or use watchlist panel |
| Enable alerts | Click bell icon on watchlist item |
| Disable alerts | Click bell icon again (turns from blue to gray) |

12.3 Data Export

| Product | Download Method |
| --- | --- |
| Static vulnerability raster | Advanced view download link |
| Maximum risk raster | Advanced view download link |
| ETA raster | Advanced view download link |
| Street forecast GeoJSON | Advanced view download link |

13. Visualization and Interpretation

13.1 Recommended Tools

| Tool | Purpose |
| --- | --- |
| QGIS | Professional raster and vector analysis |
| Leaflet (built-in) | Web-based interactive visualization |
| Python (rasterio/matplotlib) | Custom analysis and plotting |

13.2 Analysis Workflows

| Analysis | Goal Method |
| --- | --- |
| Terrain-based risk assessment | Open static_fv_10m.tif, apply color ramp |
| Temporal evolution | Animate or sequence through 48 risk_*.tif files |
| Onset timing | Query eta.tif at specific locations |
| Severity mapping | Visualize max_risk.tif with red-orange palette |
| Road impact assessment | Load street.geojson, symbolize by max_risk |

13.3 Interpretation Guidelines

| Observation | Interpretation |
| --- | --- |
| High static vulnerability + low rainfall | Low immediate risk despite vulnerable terrain |
| Moderate static + high rainfall + saturated soil | Significant dynamic risk due to environmental factors |
| ETA < 6 hours | Rapid onset requiring immediate response |
| ETA > 24 hours | More time for preparation and resource staging |
| Max risk > 0.8 | Very High category; expect severe impacts |

14. Limitations

| Limitation | Description | Impact |
| --- | --- | --- |
| Uniform rainfall assumption | Model assumes same rainfall across entire county | Localized storms may be misrepresented |
| Static soil moisture during forecast | Soil moisture fetched once, not updated across 48 hours | Saturation changes not captured |
| No hydraulic flow modeling | Does not simulate water movement across terrain | Cannot predict flow paths or depth changes |
| Resolution limited to DEM | 10-meter cells cannot represent building-scale variation | Small features may be averaged out |
| Dependent on external APIs | Open-Meteo and NASA POWER must be available | Forecasts fail if APIs are down |
| No drainage infrastructure | Storm sewers, culverts, pumps not modeled | Urban drainage effects omitted |
| Multiplicative simplification | Assumes linear independence of factors | Complex interactions not captured |
| MAX_RAIN fixed at 75 mm | Capped normalization regardless of event severity | Extreme events may be underestimated |

15. Future Work

| Enhancement | Description | Priority |
| --- | --- | --- |
| Spatial rainfall integration | Use gridded rainfall data instead of uniform assumption | High |
| Dynamic soil modeling | Update soil moisture throughout forecast window | High |
| Flood depth simulation | Add hydraulic modeling for depth predictions | Medium |
| Push notifications | WebSocket or server-sent events for real-time alerts | Medium |

16. Batch Processing

The project has one batch processing pipeline, triggered manually via the Run 48-Hour Forecast button in the Forecast Engine panel.

16.1 What the Pipeline Does

| Step | Operation | Details |
| --- | --- | --- |
| 1 | Fetch rainfall forecast | 48-hour hourly CSV from Open-Meteo (one HTTP call, 48 rows) |
| 2 | Fetch soil moisture | Today's data from NASA POWER, generates spatially-varying soil raster matching static TIF grid |
| 3 | Generate 48 risk rasters | Loop over forecast hours: risk[pixel] = static_fv[pixel] × (rain_mm/75) × soil[pixel] |
| 4 | Save outputs | 48 TIF files to data_dynamic_processed/dynamic_risk/forecast/ |
| 5 | Compute max risk | Collapse 48 rasters into single max_risk.tif (worst-case across window) |
| 6 | Compute ETA | First hour each pixel crosses 5% risk threshold → eta.tif |

16.2 Resource Requirements

| Resource | Value |
| --- | --- |
| Number of rasters | 48 * 3 |
| Size per raster | ~37 MB |
| Total disk space | ~1.78 GB |
| Processing thread | Background Python thread (server stays responsive) |
| Frontend polling | Every 2 seconds via `/api/forecast/status` |

16.3 Road Sampling as Batch Operation

When /api/roads or /api/dynamic/roads is first called:

| Action | Detail |
| --- | --- |
| Trigger | First API call after server start or cache clear |
| Operation | Sample TIF at midpoint of all 11,376 road segments |
| Terminal message | "Sampling TIF for 11376 roads..." |
| Caching | Results cached in memory for subsequent requests |

16.4 Manual Trigger

| Method | Location |
| --- | --- |
| UI button | Run 48-Hour Forecast in Forecast Engine panel |
| API endpoint | POST /api/forecast/run |
| Status check | GET /api/forecast/status returns stage, percentage, errors |

17. API

17.1 External APIs Called by the System

| API | Endpoint | Purpose | Key Details |
| --- | --- | --- | --- |
| Open-Meteo | https://api.open-meteo.com/v1/forecast | Fetches current hourly precipitation (mm/hr) and soil moisture (m³/m³ | Called every 10 minutes (cached); uses Hudson County centroid (40.73°N, 74.08°W); no API key required |
| NASA POWER | https://power.larc.nasa.gov/api/temporal/daily/point | Fetches daily soil moisture (GWETPROF parameter) | Called once per pipeline run; falls back to seasonal average if unavailable; no API key required |
| OpenStreetMap | https://nominatim.openstreetmap.org/search | Address search autocomplete | Called when user types in search bar; bounded to Hudson County; no API key required |

17.3 Scenario Definitions

| Scenario | Rainfall Rate | Description |
| --- | --- | --- |
| none | 0 mm/hr | No rainfall baseline |
| light | 2.5 mm/hr | Light rain, minimal impact |
| heavy | 15 mm/hr | Heavy rain, significant runoff |
| extreme | 75 mm/hr | MAX_RAIN threshold, maximum normalization |

18. Alert Notification

18.1 Overview

Alerts are entirely frontend-driven no push notifications or external services are involved. The system works only when the web page is open and active in the browser.

18.2 How Alerts Work

| Step | Action | Details |
| --- | --- | --- |
| 1 | User saves a location to Watchlist | Click star icon in risk popup |
| 2 | User enables alert for that location | Click bell icon on watchlist item |
| 3 | Watchlist scores refresh | On page load, mode switch, or manual refresh |
| 4 | System checks each alerted item's risk_level | Evaluates against High and Very High thresholds |
| 5 | Red notification banner appears | If any item reaches High or Very High |

18.3 Notification Banner Behavior

| Feature | Description |
| --- | --- |
| Content | Location name, risk level, and percentage |
| Position | Top of the map |
| Stacking | Up to 5 notifications at once |
| Dismissal | Each banner has a × button for individual dismissal |
| Visual indicator | Bell icon turns blue when alert is active for an item |
| Status bar | Shows alert count in Advanced view sidebar |

18.4 Alert Triggers

| Risk Level | Threshold | Alert Triggered |
| --- | --- | --- |
| None (0–20%) | < 0.20 | No |
| Low (20–40%) | 0.20–0.40 | No |
| Moderate (40–60%) | 0.40–0.60 | No |
| High (60–80%) | 0.60–0.80 | Yes |
| Very High (80–100%) | 0.80–1.00 | Yes |

18.5 Current Limitations

- Page must be open, Alerts only trigger when the page is active in browser

- No background notifications,  Closed browser or inactive tab will not receive alerts

- Refresh required,  Alerts only checked when scores are refreshed, not continuously

- No push infrastructure,  Backend does not send push notifications or emails

18.6 Production Enhancement Recommendation

For a production deployment, implement:

- WebSockets or Server-Sent Events (SSE) : Push alerts to users even when not actively    interacting

- Web Push API  :Browser notifications when page is in background

- Email/SMS gateway : Offline notification delivery

- Service Worker : Background alert processing

_____________________________________________________________________________